Experimental methods for calibration of coordinate detectors for  high energy (gammas and electrons

A.D.Belyaev1, V.M.Ionov2, V.G.Nedorezov2, N.V.Rudnev2, A.A.Turinge1
1RRC “Kurchatov Institute”, Moscow, Russia;  2Institute for Nuclear Research  RAS, Moscow, Russia.


Methods of calibration of coordinate detectors for high energy (gammas and electrons are developed. The calibration system consists of 2 thin scintillators, 4 photomultipliers, electronic modules, PC and software. Results obtained with a source of beta-particles (90Sr) are presented. The following experiment is proposed for calibrations of drift chambers in frame of the scientific program “ELISe” [1] (Electron – Ion Storage ring) using the GAMMA station at the Kurchatov synchrotron “Siberia-2” [2], which gives a possibility to study photonuclear reactions with the energies up to 2.5 GeV and realize applied physics tasks.

Each drift chamber straw tube  has a thickness of 126 microns including base polyamide KAPTON on Al (0.2 m) cowered by “C”-layer (5 m). Straw length = 60 cm; Inner diameter =  
[image: image6.png]0.3mm




   mm. Gas mixture is Ar/CO2 (80/20), HV = 1850 V. 

[image: image1.wmf]003

.

0

5

.

7

±



Experimental set-up is shown in fig.1,2.
Fig.1
Experimental set-up.

1, 2 – scintillators (polystyrene),
 dimensions : 17 х 14 x 0.3(1.5)mm;
3,4  – photomultipliers (PMT-85);

5– drift chamber;

CC – coincidence circuit.
Fig.2
[image: image2.png]


Source – (90Sr);  pmt – 
photomultiplier; sci – scintillator; 

ampl – amplifier; discr – 

discriminator; d.l.- delay line; 
c.c. – coincidence circuit;
 l.t. – level translator;
 QDC – charge-digital converter;

 crate. – CAMAC crate; Ccpc5 – 
controller.

To process information coming from the QDC unit two programs were developed which use the resources of program packages MathCad and Excel:
- ImportCCPC

- InputVariablesCCPC.

The first program shows the beta spectrum of electrons which passed the scintillator. The second one is used to comment experimental conditions.
User interface of the ImportCCPC and  InputVariablesCCPC programs is shown in fig.3.
[image: image3.png]source

slit

ampl dl.
pmt]
diser
— sci ce. It
diser
pmt
‘ ampl dl.

crate|

ccre





Fig. 3
Selection of the
channel and 

presentation of

the results.
[image: image4.jpg](o] Fle Ede Vew Insert Fomt Tods Synbokcs Window e
D-ad &5 ¥ @ v HY =
< [l B U
= TTRbleT
Clivoa teat oz table2 = READPRN("D:\Common'Pol, 5a.txt" )

v[o v

Normal

; a i a
2
3 —
i v ‘ v
ol 1 A bl =
C— ] 4
2
v Co—
i : .
st
lEvautin
3]1-5t areph
2o
1m0
5
H 50
& e

_

0 10 200 30 40 500 6w
12ex 1, 2ex 1
Chaivel

<
Press F1 for help.

READPRN("D CommenFol 3ata )

E— -
2
i 2 Get spectrum
o -
a
2
3
i 3
Spectrum obtained
from ch: el b=3
and b
12e155
w2t
K_2=10

Gam®ii: | BHE

Xy ! I G H W

[

P& oW E

2= 10

>
NUM Page 1

ko




Fig.4.

Protocol of

the experimental

conditions.

Experimental spectrum of beta-particles of 90Sr  for 0.3mm and 1.5mm scintillator thicknesses is shown in fig.5.
[image: image5.png]Matl [InputVa CPC-1]
) Fle Edt Vew Insert Fomat Tods Symbolcs Window Hep

D-FWd SRY iE=8 o« Y= Le0 | v

v[w v Bz U

Normal v [aria

,
Position in natrix M.

1- Resenv
2-Uan 1-st PMT

3-Uon 2:nd PMT

4 - koeff amplifiation 1-st PMT
5- koeff amplifiation 2-nd PMT
613 - Resenv

SR T Mm@ 63000000000
step 2 P Rn———"




Fig.5. 
Energy loss spectrum of

electrons in scintillators
of 17 mm length ( 0.3 and 
1.5 mm thickness).


As shown in fig.1, scintillator plates were situated in parallel to the beam axis. Therefore, the energy loss distributions are similar for both thickness (0.3 and 1.5 mm) of the plates. The slight difference is seen, nevertheless, which is caused by the electron scattering (escape) from the scintillators. The distributions presented in fig.5 were normalized on  the total number of events.

The result of measurements shows that the coordinate calibration of the drift chambers will be possible with accuracy of 0.3mm, at least, using the gamma beam of the storage ring “Siberia-2” at RRC “Kurchatov Institute”.

This work is supported by RFBR, grant 04-02-16996

1.  H.Simon, e.a. ,V.Lisin, A.Mushkarenkov, V.Nedorezov, A.Polonski, N.Rudnev, A.Turinge e.a. (ELISe collaboration). Technical Proposal for ELISe Experiment , GSI (2005).   
2. А.Д.Беляев, Н.В.Руднев, В.Г.Недорезов, А.А.Туринге. Фотоделение ядер-актинидов в области нуклонных резонансов: первый фотоядерный эксперимент на накопителе электронов «Сибирь-2». Ядерная физика, 67, 8  (2004) 1-5.
_1181999498.unknown

